N/AGCSU Research Programme

Phase 2 Input Dataset: Synthetic PVT, Rock-Fluid & Production Data for the UGH-1 Composite Pilot Sector

Companion documentation to: NigerDelta_PVT_RockFluid_MasterData.xlsx, NigerDelta_UGH1_Composite_Field.DATA (OPM Flow), NigerDelta_Synthetic_Production_History.xlsx

# 0. Why this dataset exists

Phase 1 of the Execution & Methodology Playbook flagged, as its top risk item, that ‘no real subsurface/PVT/history data’ exists in the public record for most Niger Delta mature fields, and instructed the team to prioritise pilot fields with at least a published analog study, reusing its reported parameters as a starting point (Playbook §7, item 2). The Phase 1 screening report itself repeats this: for the primary pilot, OML 30 (Ughelli depobelt), ‘no field-specific storage-capacity or seal-integrity study was found in the public literature-this needs to be commissioned or obtained from the operator... before Phase 2 can proceed with confidence.’

We were not able to source that operator-grade data either directly or through contacts. This package is the documented fallback the methodology itself anticipates: a fully synthetic, but literature- and analog-calibrated, dataset for a composite pilot-sector reservoir (“UGH-1”), built so Phase 2 (dynamic reservoir simulation in OPM Flow) has something concrete to run, history-match against, and design a gas-storage/EOR cycling schedule around - while being completely explicit, everywhere, about what is real, what is calibrated, and what is assumed.

Treat every number here the way the methodology itself tells you to: as representative/analog, not an operator-validated replica. Keep Phase 2–4 conclusions focused on relative comparisons (e.g. storage-with-EOR vs. storage-only) rather than presenting absolute Bscf/bbl figures as fact, and say so explicitly in your methods section.

# 1. What is genuinely real in this package

Three categories of input feed this dataset, and they carry very different evidentiary weight. Being explicit about this split is the single most important thing for your methods section.

| Category | What it is | Source |
| --- | --- | --- |
| A - Direct field data (real) | Field name, pilot ranking, published storage-capacity benchmark (488.32 / 244.16 Bscf), OML30 field list and infrastructure facts | Phase 1 deliverable (SPE/OnePetro/NUPRC/press citations already in that report) |
| B - Calibration anchor (real but of unconfirmed provenance) | Pb=Pi=2751 psia, Rsb=818 scf/STB, Bob=1.44 rb/STB, μob=0.34 cp, API=41.9, γg=0.68, T=179°F, φ=0.19, Swc=0.13, OOIP=15 MMSTB, full aquifer parameter set, base relative-permeability table, 1976-2013 monthly production/pressure history | The ‘EK6’ MBAL tank-model workbook you were given by a colleague. It behaves like a genuine field export (internally consistent monthly time series over 37 years, plausible pressure/water-influx behaviour) and every value sits inside published Niger Delta ranges (see §3) - but its origin (which real field, if any) is unverified. Used here as the single authentic anchor point. |
| C - Correlated / literature-calibrated | Full-pressure-range PVT tables (oil, gas, water), SCAL curves, per-layer petrophysics, 30x30x5 grid, 8 sub-field production histories 1976-2025 | Standard published correlations (Standing, Beggs-Robinson, Sutton, Hall-Yarborough, Lee et al.) calibrated to anchor B, with parameter ranges checked against the Niger Delta literature in §3 |

# 2. The composite pilot-sector concept

Rather than inventing an arbitrary field, this dataset represents the primary Phase-1 pilot - the OML 30 / Ughelli depobelt fields (Afiesere, Eriemu, Evwreni, Oweh, Olomoro-Oleh, Kokori, Oroni, Uzere) - as a single synthetic multi-layer sector model, “UGH-1”, with production allocated across 8 named sub-fields so the naming ties directly back to your Phase 1 report. This is explicitly a representative sector, not a claim to reproduce OML 30's true, much larger, field-wide volumes.

| Property | Value |
| --- | --- |
| Grid | 30 (I) x 30 (J) x 5 (K) cells, DX=DY=200 ft → 6,000 x 6,000 ft areal extent (826 acres) |
| Layers | 5 stacked sand layers (U1–U5), gross 35–60 ft each, NTG 0.75–0.90, φ 0.19–0.24, k 310–850 mD - a stacked-reservoir sequence in the style of the real multi-reservoir OML 30 block |
| Top depth | 8,200 ft ss (U1) to 8,625 ft ss (base U5); OWC at 8,700 ft ss |
| STOIIP | 157.6 MMSTB (volumetric, Boi = 1.44 rb/STB) |
| Aquifer | Edge, Carter-Tracy analytic - EK6 parameters used directly (thickness 77 ft, radius 16,137 ft, radius ratio 15, encroachment angle 56°, k = 566 mD) |
| Wells | 8 producers (named after the 8 real sub-fields) + 1 pre-wired, shut-in gas injector (INJ-1) for the Phase-2 storage/EOR design step |
| History window | 1976–2025 (phased sub-field start-up 1976–1981), matching the EK6 source's own 1976 start date |

# 3. PVT & rock-fluid methodology

## 3.1 Oil PVT

Solution GOR and oil FVF use the Standing (1947) correlation; oil viscosity uses Beggs-Robinson (1975). Both were selected because the Niger Delta PVT-correlation literature reviewed for this deck (see §6 sources) consistently finds Standing and Vasquez-Beggs are the correlations most often benchmarked against Niger Delta crude, and that a regionally/field calibrated correlation outperforms an uncalibrated global one - exactly the approach used here.

Each correlation is applied in its standard published form and then multiplied by a single calibration factor (per property) so that its value at the bubble point exactly reproduces the EK6 anchor. The raw (uncalibrated) correlation already lands within 3–9% of the anchor before calibration, which is a normal-sized regional gap for these correlations and supports using them for the rest of the pressure range.

| Property | Correlation (uncalibrated) at Pb | EK6 anchor | Calibration factor applied |
| --- | --- | --- | --- |
| Rs (solution GOR) | 790.4 scf/STB | 818.0 scf/STB | x 1.0349 |
| Bo (oil FVF) | 1.4367 rb/STB | 1.4400 rb/STB | x 1.0023 |
| μo (oil viscosity) | 0.3713 cp | 0.3400 cp | x 0.9156 |

The field was saturated at discovery (Pi = Pb = 2751 psia per the EK6 source), so the full pressure range of interest (114.7–2751 psia) is the saturated branch. A short undersaturated extension (2 extra points per Rs row, using Vasquez-Beggs oil compressibility for Bo and a mild power-law viscosity trend) is added purely so the simulator has a well-behaved undersaturated branch available if local pressure ever exceeds Pb during simulation (e.g. near a future gas injector).

## 3.2 Gas PVT

Pseudo-critical properties from gas gravity (0.68) via Sutton (1985); z-factor via the Hall-Yarborough (1973) explicit solution; gas viscosity via Lee, Gonzalez & Eakin (1966). CO2 (0.10%) and N2 (0.03%) are low enough that no non-hydrocarbon correction was applied. Output table spans 114.7–5,200 psia, z ranging 0.866–1.008 across that range - physically reasonable for a 0.68 SG sweet gas at 179°F.

## 3.3 Water PVT & rock compressibility

A representative Niger Delta formation-water PVT (Bw = 1.0295 rb/STB, Cw = 3.3×10⁻⁶ 1/psi, μw = 0.44 cp at 179°F, ~20,000 ppm NaCl brine) is used, since the EK6 source did not include a water PVT block. Rock compressibility (3.577×10⁻⁶ 1/psi) is taken directly from the EK6 source and used unchanged.

## 3.4 Relative permeability (SCAL)

The EK6 source's own ‘Relative Permeability’ table (three rows: residual saturation / end-point kr / Corey exponent) is used as the literal Corey-model input: Swc=0.13, Sorw=0.12, Krw_max=0.78, Kro_max=0.80, nw=3.8, no=4.0 for the water-oil system, and Sgc=0.04, Krg_max=0.77, ng=3.0 for the gas leg. The gas-oil residual oil saturation (Sorg=0.15) and oil-in-gas Corey exponent (2.0) were not given in the source and are assumed from the Niger Delta unconsolidated-sandstone SCAL literature (see §6) - these two numbers are the first to replace if real core/SCAL data ever becomes available. Weak Corey-shaped capillary pressure curves (5 psi at Swc, decreasing to 0) were added since unconsolidated, high-permeability Niger Delta sand typically shows low capillary entry pressure.

# 4. Petrophysics & grid ranges vs. published Niger Delta data

Every range used in the synthetic grid was checked against the literature rather than invented freely:

- Porosity 0.19–0.24 used here vs. published Niger Delta ranges of 0.105–0.309 (well-log studies), 0.18–0.34 (unconsolidated core studies), 0.238–0.241 (Eni field core data) - comfortably inside range.

- Permeability 310–850 mD used here vs. published range 35–20,000 mD for unconsolidated Niger Delta sands - a conservative mid-range choice, not the high end.

- Connate water saturation 0.13–0.20 used here vs. published range 0.035–0.43 - inside range, biased toward the lower/moderate end consistent with the EK6 anchor (0.13).

- API 41.9 (single value, from anchor) vs. published range 20.5–41.9 API across Niger Delta fields - sits at the high (light-oil) end of the published range, which is a real feature of the anchor data, not an error; flagged here so you can judge whether it is representative enough of OML 30 specifically or should be shaded down in a future revision.

- Reservoir temperature 179°F vs. published range 170–180°F - matches closely.

- Initial pressure 2,751 psia vs. published range 2,328–2,553 psia (Eni field) and 1,500–3,000 psia (general Niger Delta) - slightly above the Eni-field range but inside the wider Niger Delta range.

# 5. Production history dataset

NigerDelta_Synthetic_Production_History.xlsx contains monthly oil/gas/water rates, cumulatives, reservoir pressure, solution GOR and water cut for the 8 named sub-fields, 1976–2025 (4,596 rows). Each sub-field's oil rate follows a build-up → plateau → Arps hyperbolic-decline curve, scaled so cumulative production converges on an estimated ultimate recovery (STOIIP x recovery factor, 20–33% per sub-field). Sub-field STOIIP is allocated so the sector total (157.6 MMSTB) matches the OPM Flow grid's own volumetric STOIIP exactly - the two files are internally consistent with each other by construction.

The shape (long period of near-flat, aquifer-supported pressure, then rising water cut, softening plateau, extended stripper-well tail) mirrors the real behaviour visible in the EK6 source's own 1976–2013 monthly series, extrapolated across 8 sub-fields to 2025. No public, field-specific monthly production series for OML 30's named fields was located in the literature - this is a sized-and-shaped synthetic stand-in for exercising your Phase-2 history-matching workflow, not a source of real reserves numbers.

# 6. OPM Flow deck (NigerDelta_UGH1_Composite_Field.DATA)

A single, self-contained black-oil deck (RUNSPEC / GRID / PROPS / REGIONS / SOLUTION / SUMMARY / SCHEDULE) built directly from the tables above:

- GRID: 30x30x5 box grid, per-layer DZ/NTG/PORO/PERMX/PERMY/PERMZ via EQUALS, with a mild areal permeability multiplier so the sector is not a perfect layer-cake.

- PROPS: the exact PVTO / PVDG / PVTW / ROCK / DENSITY / SWOF / SGOF tables documented above.

- SOLUTION: EQUIL at Pi=Pb=2751 psia (fully saturated, no initial gas cap), OWC at 8,700 ft.

- SCHEDULE: AQUCT + AQUANCON wiring the EK6-derived aquifer to the sector's edge; WELSPECS/COMPDAT for the 8 named producers; 12 DATES/WCONPROD control blocks (1976–2025) whose field-total rate targets are taken directly from the production workbook; and a pre-wired, shut-in gas injector (INJ-1) at 1-Jan-2026 for the Phase-2 team to design the cyclic injection–withdrawal (storage) and gas-flood EOR schedule against, per the Execution Methodology's own Phase 2, Step 4 instruction.

Before relying on simulation output, run the deck once, check OPM Flow's own material-balance/convergence report (the Execution Methodology explicitly warns that cyclic injection–withdrawal stresses black-oil formulations at timestep transitions - § Phase 2 Gotchas), and treat the first run as a smoke test of the workflow, not a finished history match.

# 7. Limitations to state explicitly in your methods section

- This is a synthetic, analog-calibrated dataset, not operator-supplied field data. No real 3D seismic, well logs, cored SCAL, or metered production for OML 30's named fields underlie it.

- The EK6 anchor's own provenance (which real field, if any, it was exported from) is unconfirmed - it is used because it is the only internally-consistent, Niger-Delta-plausible dataset available, not because its origin has been verified.

- STOIIP (157.6 MMSTB) is a sector-scale volume for a single synthetic fault block, not a claim about OML 30's true field-wide reserves.

- Two SCAL parameters (Sorg, oil-in-gas Corey exponent) are literature-typical assumptions, not sourced from the anchor or any Niger Delta core study specific to this field.

- The OSO field's published storage-capacity figures (488.32 / 244.16 Bscf) are kept as an external literature benchmark only - they are not re-derived from this dataset's own PVT/petrophysics and should not be presented as validating this sector model.

- Keep Phase 2–4 conclusions comparative (with-EOR vs. without, storage vs. no-storage) rather than reporting single absolute numbers as certain, exactly as the Execution Methodology instructs for an approximation/analog model.

# 8. Sources consulted

- Standing, M.B. (1947), A Pressure-Volume-Temperature Correlation for Mixtures of California Oils and Gases.

- Beggs, H.D. & Robinson, J.R. (1975), Estimating the Viscosity of Crude Oil Systems, JPT.

- Sutton, R.P. (1985), pseudocritical property correlation for natural gases.

- Hall-Yarborough (1973), explicit z-factor correlation.

- Lee, A.L., Gonzalez, M.H., Eakin, B.E. (1966), The Viscosity of Natural Gases, JPT.

- “Black Oil Empirical PVT Correlations Screening for the Niger Delta Crude”, ResearchGate.

- “Bubble point Pressure Correlations for Niger Delta Crude Oils”, ResearchGate.

- “Integrated reservoir management in a Niger Delta oil rim field”, Cogent Engineering (Tandfonline).

- “Reservoir description and characterization of Eni field Offshore Niger Delta, southern Nigeria”, J. Petroleum Exploration and Production Technology (Springer).

- “Determination of porosity-permeability correlation with pressure and depth in part of Niger Delta basin, Nigeria”, Earth Science Malaysia (2021).

- “Experimental Determination of Relative Permeability from Unconsolidated Core Samples of The Niger Delta”, SPE-172478-MS, OnePetro.

- “Influence of Well-Pattern Geometry on ASP Flooding Performance in a Heterogeneous Niger Delta Sandstone Reservoir”, Research Square.

- This programme's own Phase 1 deliverable (“Phase 1 - National Field Inventory & Screening.docx”) and Execution & Methodology Playbook (“NAGCSU_Execution_Methodology.docx”).

- User-supplied “RESERVOIR_EK6_INFO.xlsx” (MBAL tank-model export) - the calibration anchor for this entire package.